Cold Weather Concrete & Masonry:

- Temperature determined per local weather in the specific area / municipality.

- When temperature is 40 degrees and below for 24 hours, cold weather concrete precautions may be required. Consult with inspector of record to verify specific requirements which may include:

  - Protection of soils

  - Heated blankets or other heat retention means

  - Concrete additives

- When temperature is below 28 degrees, concrete placement may be suspended. Consult with inspector of records.

- Concrete only approved for same day pour.

- Deck piers, may be poured if full depth below grade, have insulated coverage at grade including immediate surround and no observed frost.

- Manufactured stone, follow manufactures specifications.

- Cut sheets shall be submitted for all accelerants used. Approval by the structural engineer of record for all accelerants if commercial. Ambient temperature must be equal to or greater than what is indicated on the cut sheet.